SNMP OID

This document is a reference material that systematically organizes OID (Object Identifier) information required when configuring SNMP GET collectors.

Understanding OID Structure

OID Tree Structure

SNMP OIDs have a hierarchical tree structure where each node inherits a unique number from its parent node. This is similar to a file system directory structure.

Hierarchical Structure from Root Node

1              - ISO (International Organization for Standardization)
└── 1.3        - org (ISO identified organization)
    └── 1.3.6  - dod (US Department of Defense)
        └── 1.3.6.1 - internet (Internet)
            ├── 1.3.6.1.1   - directory (X.500 Directory)
            ├── 1.3.6.1.2   - mgmt (Management)
            │   └── 1.3.6.1.2.1 - mib-2 (MIB-II, standard management objects)
            ├── 1.3.6.1.3   - experimental (experimental objects)
            ├── 1.3.6.1.4   - private (private)
            │   └── 1.3.6.1.4.1 - enterprises (enterprise-specific private objects)
            ├── 1.3.6.1.5   - security (security)
            ├── 1.3.6.1.6   - snmpV2 (SNMPv2)
            ├── 1.3.6.1.7   - mail (mail)
            └── 1.3.6.1.8   - features (features)

Role and Management Authority of Each Branch

| OID | Name | Role | Authority | Usage Example |
| --- | --- | --- | --- | --- |
| 1.3.6.1.2.1 | mib-2 | Standard MIB objects | IANA/IETF | System info, interface statistics |
| 1.3.6.1.4.1 | enterprises | Vendor-specific objects | Companies | Cisco: 1.3.6.1.4.1.9 |
| 1.3.6.1.3 | experimental | Experimental objects | IANA | Pre-standardization testing |
| 1.3.6.1.6 | snmpV2 | SNMPv2-specific objects | IANA | SNMPv2 modules, domains |

Benefits of Hierarchical Structure

Uniqueness Guarantee: Each OID is a globally unique identifier

Scalability: New nodes can be added to existing structure

Management Efficiency: Clear separation of management authority per branch

Standardization: Systematic standard management through RFC documents

Real OID Example Interpretation

Example: 1.3.6.1.2.1.1.1.0 (system description)

1       - ISO root
1.3     - ISO identified organization
1.3.6   - US Department of Defense
1.3.6.1 - Internet
1.3.6.1.2 - Management objects
1.3.6.1.2.1 - MIB-II standard
1.3.6.1.2.1.1 - system group
1.3.6.1.2.1.1.1 - sysDescr object
1.3.6.1.2.1.1.1.0 - scalar instance (single value)

Example: 1.3.6.1.4.1.9.2.1.56.0 (Cisco CPU utilization)

1.3.6.1.4.1 - Enterprise private branch
1.3.6.1.4.1.9 - Cisco Systems (enterprise number: 9)
1.3.6.1.4.1.9.2.1.56.0 - avgBusy5 (5-minute average CPU utilization)

Notation Methods

Numeric Notation (used in Sonar collectors):

1.3.6.1.2.1.1.1.0

Name Notation (used in MIB files):

iso.org.dod.internet.mgmt.mib-2.system.sysDescr.0

Major Standard OIDs

System Group (1.3.6.1.2.1.1)

OIDs that provide basic system information.

| OID | Name | Description | Data Type | Example Value |
| --- | --- | --- | --- | --- |
| 1.3.6.1.2.1.1.1.0 | sysDescr | System description | string | Cisco IOS Software |
| 1.3.6.1.2.1.1.2.0 | sysObjectID | System object ID | OID | 1.3.6.1.4.1.9.1.1 |
| 1.3.6.1.2.1.1.3.0 | sysUpTime | System uptime | 32-bit int | 12345678 (unit: centiseconds) |
| 1.3.6.1.2.1.1.4.0 | sysContact | Administrator contact | string | admin@company.com |
| 1.3.6.1.2.1.1.5.0 | sysName | System name | string | router01.company.com |
| 1.3.6.1.2.1.1.6.0 | sysLocation | System location | string | Seoul Data Center |

Interfaces Group (1.3.6.1.2.1.2)

OIDs that provide network interface information.

Basic Interface Information

| OID | Name | Description | Data Type |
| --- | --- | --- | --- |
| 1.3.6.1.2.1.2.1.0 | ifNumber | Total number of interfaces | 32-bit int |
| 1.3.6.1.2.1.2.2.1.1.X | ifIndex | Interface index | 32-bit int |
| 1.3.6.1.2.1.2.2.1.2.X | ifDescr | Interface description | string |
| 1.3.6.1.2.1.2.2.1.3.X | ifType | Interface type | 32-bit int |
| 1.3.6.1.2.1.2.2.1.4.X | ifMtu | Maximum transmission unit | 32-bit int |
| 1.3.6.1.2.1.2.2.1.5.X | ifSpeed | Interface speed | 32-bit int |

Traffic Statistics (32-bit Counters)

| OID | Name | Description | Data Type |
| --- | --- | --- | --- |
| 1.3.6.1.2.1.2.2.1.10.X | ifInOctets | Received bytes | 32-bit int |
| 1.3.6.1.2.1.2.2.1.11.X | ifInUcastPkts | Received unicast packets | 32-bit int |
| 1.3.6.1.2.1.2.2.1.12.X | ifInNUcastPkts | Received non-unicast packets | 32-bit int |
| 1.3.6.1.2.1.2.2.1.13.X | ifInDiscards | Received dropped packets | 32-bit int |
| 1.3.6.1.2.1.2.2.1.14.X | ifInErrors | Received error packets | 32-bit int |
| 1.3.6.1.2.1.2.2.1.16.X | ifOutOctets | Transmitted bytes | 32-bit int |
| 1.3.6.1.2.1.2.2.1.17.X | ifOutUcastPkts | Transmitted unicast packets | 32-bit int |
| 1.3.6.1.2.1.2.2.1.18.X | ifOutNUcastPkts | Transmitted non-unicast packets | 32-bit int |
| 1.3.6.1.2.1.2.2.1.19.X | ifOutDiscards | Transmitted dropped packets | 32-bit int |
| 1.3.6.1.2.1.2.2.1.20.X | ifOutErrors | Transmitted error packets | 32-bit int |

Note: X is the interface index number. Example: 1.3.6.1.2.1.2.2.1.10.1 (received bytes of first interface)

High Capacity Counters (1.3.6.1.2.1.31)

64-bit counters for high-speed interfaces.

| OID | Name | Description | Data Type |
| --- | --- | --- | --- |
| 1.3.6.1.2.1.31.1.1.1.1.X | ifName | Interface name | string |
| 1.3.6.1.2.1.31.1.1.1.6.X | ifHCInOctets | Received bytes (64-bit) | 64-bit int |
| 1.3.6.1.2.1.31.1.1.1.7.X | ifHCInUcastPkts | Received unicast packets (64-bit) | 64-bit int |
| 1.3.6.1.2.1.31.1.1.1.8.X | ifHCInMulticastPkts | Received multicast packets (64-bit) | 64-bit int |
| 1.3.6.1.2.1.31.1.1.1.9.X | ifHCInBroadcastPkts | Received broadcast packets (64-bit) | 64-bit int |
| 1.3.6.1.2.1.31.1.1.1.10.X | ifHCOutOctets | Transmitted bytes (64-bit) | 64-bit int |
| 1.3.6.1.2.1.31.1.1.1.11.X | ifHCOutUcastPkts | Transmitted unicast packets (64-bit) | 64-bit int |
| 1.3.6.1.2.1.31.1.1.1.12.X | ifHCOutMulticastPkts | Transmitted multicast packets (64-bit) | 64-bit int |
| 1.3.6.1.2.1.31.1.1.1.13.X | ifHCOutBroadcastPkts | Transmitted broadcast packets (64-bit) | 64-bit int |

Host Resources MIB (1.3.6.1.2.1.25)

OIDs that provide system resource information.

System Information

| OID | Name | Description | Data Type |
| --- | --- | --- | --- |
| 1.3.6.1.2.1.25.1.1.0 | hrSystemUptime | System uptime | 32-bit int |
| 1.3.6.1.2.1.25.1.2.0 | hrSystemDate | System date/time | string |
| 1.3.6.1.2.1.25.1.3.0 | hrSystemInitialLoadDevice | Boot device | 32-bit int |
| 1.3.6.1.2.1.25.1.4.0 | hrSystemInitialLoadParameters | Boot parameters | string |

Memory Information

| OID | Name | Description | Data Type |
| --- | --- | --- | --- |
| 1.3.6.1.2.1.25.2.2.0 | hrMemorySize | Total memory size (KB) | 32-bit int |
| 1.3.6.1.2.1.25.2.3.1.3.X | hrStorageSize | Storage size | 32-bit int |
| 1.3.6.1.2.1.25.2.3.1.6.X | hrStorageUsed | Used storage | 32-bit int |

CPU Information

| OID | Name | Description | Data Type |
| --- | --- | --- | --- |
| 1.3.6.1.2.1.25.3.3.1.2.X | hrProcessorLoad | CPU utilization (%) | 32-bit int |

Vendor-Specific Private Enterprise OIDs

Cisco Systems (1.3.6.1.4.1.9)

Major Cisco OIDs:

1.3.6.1.4.1.9.2.1.56.0    - avgBusy5 (5-minute average CPU utilization)
1.3.6.1.4.1.9.2.1.57.0    - avgBusy1 (1-minute average CPU utilization)
1.3.6.1.4.1.9.9.48.1.1.1.5.X - cpmCPUTotal5min (CPU 5-minute average)

HP Enterprise (1.3.6.1.4.1.11)

Major HP OIDs:

1.3.6.1.4.1.11.2.14.11.5.1.9.6.1.0 - hpSwitchCpuStat (CPU statistics)

Net-SNMP (1.3.6.1.4.1.8072)

Major Net-SNMP OIDs:

1.3.6.1.4.1.8072.1.3.2.3.1.2.X - ssSwapIn (swap usage)
1.3.6.1.4.1.8072.1.3.2.4.1.2.X - ssCpuUser (user CPU)

Practical Usage Guide

1. OID Discovery Methods

Linux/macOS

Installing Net-SNMP Tools:

Ubuntu/Debian systems:

sudo apt-get install snmp snmp-mibs-downloader

CentOS/RHEL systems:

sudo yum install net-snmp-utils

macOS (Homebrew):

brew install net-snmp

Basic discovery commands:

Query all system information:

snmpwalk -v2c -c public 192.168.1.1 1.3.6.1.2.1.1

Query interface information:

snmpwalk -v2c -c public 192.168.1.1 1.3.6.1.2.1.2.2.1.2

Query specific OID value:

snmpget -v2c -c public 192.168.1.1 1.3.6.1.2.1.1.1.0

Display in table format:

snmptable -v2c -c public 192.168.1.1 1.3.6.1.2.1.2.2

Windows

Method 1: Net-SNMP for Windows

Download and Installation:

Download from Net-SNMP Windows Binaries

Use in Command Prompt (CMD) or PowerShell after installation

Command Usage (same as Linux):

Query all system information:

snmpwalk -v2c -c public 192.168.1.1 1.3.6.1.2.1.1

Query interface information:

snmpwalk -v2c -c public 192.168.1.1 1.3.6.1.2.1.2.2.1.2

Query specific OID value:

snmpget -v2c -c public 192.168.1.1 1.3.6.1.2.1.1.1.0

Method 2: PowerShell SNMP Module

Install SNMP Module:

Install SNMP module from PowerShell Gallery:

Install-Module -Name SNMP -Force

PowerShell Commands:

Query specific OID value:

Get-SnmpData -IP "192.168.1.1" -Community "public" -OID "1.3.6.1.2.1.1.1.0"

OID Walk (PowerShell script example) - Query system information:

$target = "192.168.1.1"
   $community = "public"
   $baseOid = "1.3.6.1.2.1.1"

   1..6 | ForEach-Object {
       $oid = "$baseOid.$_.0"
       try {
           $result = Get-SnmpData -IP $target -Community $community -OID $oid -UDPport 161
           Write-Host "$oid : $($result.Data)"
       }
       catch {
           Write-Host "$oid : Error - $($_.Exception.Message)"
       }
   }

Method 3: GUI Tools

[ManageEngine MibBrowser](https://www.manageengine.com/products/mibbrowser-free-tool/): Free MIB browser

[iReasoning MIB Browser](https://ireasoning.com/): Commercial MIB browser

Recommended Installation Path:

Developers/IT Administrators: Net-SNMP for Windows (prefer command-line tools)

PowerShell Users: PowerShell SNMP module

General Users: GUI tools (MIB Browser, etc.)

2. SNMP GET Collector Configuration Examples

System monitoring:

1.3.6.1.2.1.1.1.0=system_description,1.3.6.1.2.1.1.3.0=uptime,1.3.6.1.2.1.25.1.1.0=hr_uptime

Interface traffic monitoring:

1.3.6.1.2.1.2.2.1.10.1=if1_in_octets,1.3.6.1.2.1.2.2.1.16.1=if1_out_octets,1.3.6.1.2.1.31.1.1.1.6.1=if1_hc_in_octets

CPU and memory monitoring:

1.3.6.1.2.1.25.3.3.1.2.1=cpu_load,1.3.6.1.2.1.25.2.2.0=memory_size,1.3.6.1.2.1.25.2.3.1.6.1=memory_used

3. 32-bit vs 64-bit Counter Selection

| Interface Speed | Recommended Counter | Reason |
| --- | --- | --- |
| ≤ 20 Mbps | 32-bit | Sufficient wrap around time |
| 20 Mbps ~ 650 Mbps | 64-bit (Octets), 32-bit (Packets) | Bytes increase rapidly |
| ≥ 650 Mbps | 64-bit | All counters increase rapidly |

4. Counter Wrap Around Time Calculation

32-bit counter maximum value: 4,294,967,295

Wrap Around time examples:

10 Mbps: approximately 57 minutes

100 Mbps: approximately 5.7 minutes

1 Gbps: approximately 34 seconds

For high-speed networks, always use 64-bit counters (ifHCInOctets, ifHCOutOctets).

Online Reference Materials

Official Standard Documents

[RFC 1155](https://tools.ietf.org/html/rfc1155): Structure and Identification of Management Information

[RFC 1213](https://tools.ietf.org/html/rfc1213): Management Information Base for Network Management (MIB-II)

[RFC 2578](https://tools.ietf.org/html/rfc2578): Structure of Management Information Version 2 (SMIv2)

[RFC 2233](https://tools.ietf.org/html/rfc2233): The Interfaces Group MIB using SMIv2

OID Databases

[Object Identifier Repository](https://oid-base.com/): Comprehensive OID database and search

[IANA Structure of Management Information](https://www.iana.org/assignments/smi-numbers/): Official SMI numbers and MIB-II registry

[IANA Private Enterprise Numbers](https://www.iana.org/assignments/enterprise-numbers/): Official enterprise number registry

[MonitorTools MIB Collection](https://www.monitortools.com/tech/snmp/mib/): Vendor-specific MIB file collection

Vendor-Specific MIB Information

[Cisco SNMP Object Navigator](https://snmp.cloudapps.cisco.com/Support/SNMP/do/BrowseMIB.do): Cisco MIB object search

[Juniper MIB Explorer](https://apps.juniper.net/mib-explorer/): Juniper device MIB information

This reference material enables effective SNMP GET collector configuration and collection of various information from network devices.